[image: image2.wmf]                     Jointcom                               Combiner 
[image: image3.png]J5int|:9m



                   JCDUP-8024C-R1B 
· High Q cavity structure
· Indoor application 
· Good VSWR
· Light weight 
· Average power rating 100W 
· Custom design available 
· DC Stop 
· High Stability
Technical Data：

	Channels
	C/G
	DCS
	3G

	Frequency Range(MHz)
	800~960
	1710~1880
	1920~2170

	Band Width(MHz)
	160
	170
	250

	Insert Loss(dB)
	≤0.3
	≤0.6
	≤0.6

	Ripple in-band(dB)
	≤0.4
	≤0.4
	≤0.4

	Stop-band Attenuation(dB)
	≥80@CH2

≥80@CH3
	≥80@CH1

≥80@CH3
	≥80@CH2

≥80@CH1

	Isolation(dB)
	≥80@CH2

≥80@CH3
	≥80@CH1

≥80@CH3
	≥80@CH2

≥80@CH1

	Return Loss(dB)
	≥18

	Impedance(Ω)
	50

	PIM (dBc)
	≤-140@+43dBm×2

	Connectors
	N-F

	Power Rating(W)
	 100

	Temperature(℃)
	-25~+65

	Size(mm)
	224×146×40

	Weight(kg)
	1.73


[image: image4.png]


Dimension：

[image: image1.png]



� EMBED AutoCAD.Drawing.16  ���








Add:4/F No.92 Huaxing Road,Hangzhou, China Tel：+86-571-87381700  Fax：+86-571-88911556 http: www.jointcom.com

[image: image5.wmf]_1289131010.dwg

